Required Qualifications of the Special Inspector

| Special Inspection Category | Code Reference | Primary Inspector or Inspection Supervisor Qualifications | Supplemental Inspector Qualifications (under the direct supervision of the Inspection Supervisor) |
| --- | --- | --- | --- |
| Verification of Fabricators | 1704.2.5; 1704.2.5.1 | See below for type of fabrication |  |
| Special Cases | 1705.1.1 | As identified by the Engineer of Record, on a case by case, in the Statement of Special Inspections. At a minimum, the Special Inspector, or their Agent, shall have 5-years related work experience on projects of comparable size and complexity. A written project list with references shall be provided. |  |
| Steel Construction and High Strength Bolting | 1705.2 Exception; 1705.2.1; Table 1705.7 Item 5 | Registered professional engineer or architect or Current ICC Structural Steel and Bolting Special Inspector Certificate plus one year of related experience | Engineer-In-Training (EIT) with one year of related experience |
| Welding | 1705.2 Exception; 1705.2.1; 1705.2.2; Table 1705.2.3 Item 1.a; Table 1705.3 Item 2; 1705.4 (welding of reinforcement) | Registered professional engineer or architect or Current ICC Structural Welding Special Inspector Certificate plus one year of related experience or Current AWS Senior Certified Welding Inspector or Current AWS Certified Welding Inspector with AWS D1.1 endorsements or Current NDT Level III Certificate | Engineer-In-Training (EIT) with one year of related experience or Current ICC Structural Steel and Bolting Inspector Certificate plus one year of related experience or Current Certified Associate Welding Inspector plus one year of related experience or Current NDT Level II Certificate plus one year of related experience |
| Cold-Formed Steel Deck | 1705.2 Exception; 1705.2.2 | Registered professional engineer or architect or Current ICC Structural Steel and Bolting Inspector Certificate plus one year of related experience See “Welding” for additional requirements. | Engineer-In-Training (EIT) with one year of related experience See “Welding” for additional requirements. |

| Open-Web Steel Joists & Joist Girders | 1705.2.3; Table 1705.2.3 | Registered professional engineer or architect or Current ICC Structural Steel and Bolting Inspector Certificate plus one year of related experience See “Welding” for additional requirements. | Engineer-In-Training (EIT) with one year of related experience See “Welding” for additional requirements. |
| --- | --- | --- | --- |
| Cold Formed Steel Framing | 1705.2.4 | Registered professional engineer or architect or Current ICC Structural Steel and Bolting Inspector Certificate plus one year of related experience or Current ICC Structural Welding Special Inspector Certificate plus one year of related experience | Engineer-In-Training (EIT) with one year of related experience |
| Concrete Construction | 1705.3; Table 1705.3 Items 1, 3, 4, 5, 6, 7, 8, and 12; Table 1705.8 Item 3 | Registered professional engineer or architect or Current ICC Reinforced Concrete Special Inspector Certificate and ACI Concrete Field Testing Technician Certificate, Grade 1 or Current ACI Concrete Construction Special Inspector Certificate or Current NICET Concrete Technician Level III Certificate in Construction Materials Testing | Engineer-In-Training (EIT) with one year of related experience or Current Associate ACI Concrete Construction Special Inspector with one year of related experience or Current NICET Concrete Technician Level II Certificate in Construction Materials Testing with one year of related experience |
| Prestressed Concrete Construction (see also Concrete Construction) | Table 1705.3 Items 1, 9, and 10; Table 1705.7 Item 6 | Registered professional engineer or Current ICC Pre-stressed Concrete Special Inspector Certificate with one year of related experience or Current PCI Quality Control Technician / Inspector Level II certificate with one year of related experience | Engineer-In-Training (EIT) with one year of related experience or Current PCI Quality Control Technician / Inspector Level I certificate with one year of related experience |

| Post-tensioned Concrete Construction (see also Concrete Construction) | 1705.3 Exception 3; Table 1705.3 Items 1, 9, and 11 | Registered professional engineer or Current PTI Level 2 Unbonded PT Inspector Certificate or Current ICC Prestressed Concrete Special Inspector Certificate with one year of related experience | Engineer-In-Training (EIT) with one year of related experience or Current PTI Level 1 Unbonded PT Inspector Certificate with one year of related experience |
| --- | --- | --- | --- |
| Masonry Construction | 1705.4 | Registered professional engineer or architect or Current ICC Structural Masonry Special Inspector Certificate plus one year of related experience | Engineer-In-Training (EIT) with one year of related experience |
| Wood Construction | 1705.5 | Registered professional engineer or architect or Current ICC Commercial Building Inspector Certificate plus one year of related experience | Engineer-In-Training (EIT) with one year of related experience |
| Verification of Site Soil Condition, Fill Placement, and Load-Bearing Requirements | 1705.6; Table 1705.6; 1804.6 Exception | Current NICET Soils Technician Level II Certificate in Construction Materials Testing or Current ICC Soils Special Inspector Certificate plus one year of related experience or Engineer-In-Training (EIT) with one year of related experience or Geologist-In-Training (GIT) with one year of related experience | Current NICET Soils Technician Level I Certificate in Construction Materials Testing plus one year of related experience |
| Deep Foundations | 1705.7; Table 1705.7; 1705.8; Table 1705.8; 1705.9 | Current NICET Soils Technician Level II Certificate in Construction Materials Testing or Engineer-In-Training (EIT) with one year of related experience or Geologist-In-Training (GIT) with one year of related experience | Current NICET Soils Technician Level I Certificate in Construction Materials Testing plus one year of related experience |
| Rammed Aggregate Piers & Stone Columns | 1705.1.1 | Current NICET Soils Technician Level II Certificate in Construction Materials Testing or Engineer-In-Training (EIT) with one year of related experience or Geologist-In-Training (GIT) with one year of related experience | Current NICET Soils Technician Level I Certificate in Construction Materials Testing plus one year of related experience |
| Sprayed Fire-Resistant Material | 1705.14 | Registered professional engineer or architect with one year of related experience or Current ICC Spray-Applied Fireproofing Special Inspector Certificate | Engineer-In-Training (EIT) with one year of related experience or Current ICC Fire Inspector I Certificate plus one year of related experience |
| Mastic and Intumescent Fire-Resistant Coatings | 1705.15 | Registered professional engineer or architect with one year of related experience or Current ICC Spray-Applied Fireproofing Special Inspector Certificate | Engineer-In-Training (EIT) with one year of related experience or Current ICC Fire Inspector I Certificate plus one year of related experience |
| Exterior Insulation and Finish System | 1705.16 | Registered professional engineer or architect or Current AWCI EIFS Inspector Certificate | Engineer-In-Training (EIT) with one year of related experience |
| Fire-Resistant Penetrations and Joints | 1705.17 | Registered professional engineer or architect with one year of related experience or Current ICC Fire Inspector I Certificate plus one year of related experience | EIT with one year of related experience |
| Smoke Control | 1705.18 | Registered Professional Engineer (Mechanical or Fire Protection Discipline) and Certification as Air Balancer or Current AABC Technician Certification with one year of related experience | Engineer-In-Training (EIT) with one year of related experience |
| Wind Resistance | 1705.11 | As recognized above for each specific type of component or material. |  |
| Seismic Resistance | 1705.12; 1705.13 | As recognized above for each specific type of component or material. |  |